9.1 Progression of Planned Technologies

| Technology | Innovation/Technology Challenge | State-of-the-art (SoA) | Innovation Beyond SoA |
| --- | --- | --- | --- |
| Highly efficient and ultra power dense electric motor TRL Start: 3 TRL Finish: 6 | Aerospace propulsion requires very high gravimetric power density while maintaining efficiency and compactness. Increasing output intensifies rotor stress, partial discharge risk, and thermal bottlenecks. At altitude, cooling efficiency drops and any excess mass in windings, housings, or filters reduces payload and range. The challenge is to deliver high continuous power in a compact, lightweight design without compromising reliability. | State-of-the-art solutions mainly use permanent magnet synchronous machines with laminated cores and liquid cooling. Siemens achieved ~5 kW/kg at 11 krpm, Honeywell ~8 kW/kg at 19 krpm, NASA prototypes ~10–13 kW/kg at 20 krpm, and EMRAX axial-flux machines ~10 kW/kg. These approaches improve density but are limited by bulk copper windings, restricted slot fill, and thermal management challenges, especially at high currents. | SkyDrive’s machine achieves ~13 kW/kg at 100 kW, 1 kV, 18,000 rpm with 95–98% efficiency. Additive-manufactured housings improve thermal conductivity by 30% and cut temperatures by 25%. Hollow conductors enable 25 A/mm² with direct liquid cooling. Spray cooling addresses hotspots, while amorphous laminations reduce losses by 30%. Carbon-fibre rotor sleeves (>3 GPa) contain stresses at speed. These yield ~24% higher density and up to 47 kg system mass savings, enabling 10–15% more range. KPI: ≥10% propulsion system mass reduction, 13 kW/kg motor power density, and +10–15% aircraft range/payload improvement. |
| Reliability and Redundancy through Multiphase Design TRL Start: 3 TRL Finish: 6 | Reliability is paramount for electric aircraft propulsion, yet conventional three-phase systems present a single point of failure: one winding or inverter fault can shut down the drivetrain. Redundancy achieved through duplicating hardware adds significant weight and complexity. The challenge is to build redundancy into the motor-drive itself, maintaining safe operation under faults without compromising density. | Most demonstrators employ three-phase permanent magnet motors with two-level inverters. Redundancy is often handled by modular three-phase units or multi-channel drives, but this adds mass and complexity. Rolls-Royce starter-generators use conservative liquid-cooled three-phase PM designs, and EMRAX axial-flux motors follow a similar convention. Multiphase topologies have been studied in academia, but few have reached aerospace maturity. | SkyDrive introduces six-phase and dual three-phase machines with integrated multilevel, fault-tolerant inverters. Dual three-phase topology achieves superior harmonic cancellation and reduced magnetic saturation, maintaining density even in degraded mode. Torque ripple falls by 50–80%, common-mode voltage by 40–60%, and operational life extends >30% through reduced vibration and insulation stress. Most critically, the motor continues operation under partial winding or phase loss. KPI: 100% improvement in fault tolerance vs. SoA |
| Altitude-Resilient Multilevel Drive for Partial Discharge Mitigation TRL Start: 3 TRL Finish: 6 | At high altitude, reduced air pressure lowers the dielectric strength of insulation systems, making windings and converter components more susceptible to partial discharge (PD). In motors, steep voltage gradients between turns cause localised breakdown and progressive insulation erosion. In drives, high dv/dt switching and overshoot from wide-bandgap (WBG) devices generate strong electric field stresses that initiate PD. These events accumulate into dielectric breakdown, leading to one of the most critical failure modes in aerospace electrification. | Conventional aerospace solutions mitigate PD risk by applying thicker insulation layers, reinforced slot liners, and conservative voltage limits (<800 V). In drives, bulky filters and snubber circuits are used to smooth transitions. While effective short-term, these strategies add significant weight, reduce gravimetric density, and are not scalable to >1 kV systems. Research demonstrators (e.g. Siemens, NASA) have investigated partial discharge testing in motors and converters, but solutions remain fragmented, with separate protection for motor windings and inverter insulation. | SkyDrive addresses PD holistically across motor and drive. Multilevel inverters generate stepped waveforms, reducing dv/dt by ~50% and suppressing overshoot by >90%, lowering stress at winding interfaces. In parallel, advanced winding insulation with amorphous laminations, improved slot fillers, and optimised phase-to-phase spacing reduce localised electric fields. Additive-manufactured housings integrate cooling directly at hotspots, lowering thermal stress that accelerates PD. Combined, these measures extend insulation lifespan 2–3× compared to SoA, delivering reliable 1 kV operation at altitude. KPI Statement: 2–3× insulation life improvement, 50–70% EMI/voltage stress reduction, and validated PD suppression |
| Technology | Innovation/Technology Challenge | State-of-the-art (SoA) | Innovation Beyond SoA |
| Power-on-Chip Converter with Integrated Cooling for Compactness TRL Start: 3 TRL Finish: 6 | Converters must deliver very high power density in minimal volume. Discrete devices, heavy busbars, and external passives create parasitics, switching losses, and poor thermal uniformity. Redundancy strategies often add more weight, directly penalising gravimetric efficiency. | Today’s aerospace converters (e.g. Evolito, Helix UK) use SiC MOSFET modules with external passives and cold-plate liquid cooling. While efficient, these systems remain bulky, parasitic-limited, and constrained to ~50–100 W/cm² thermal flux removal. Their scalability is poor, restricting modular architectures. | SkyDrive develops Power-on-Chip converters integrating wide-bandgap switches, drivers, and passives on hybrid substrates. Embedded passives halve component count and reduce parasitic inductance, enabling higher switching frequencies. Additive-manufactured microchannels in substrates achieve >1000 W/cm² thermal flux dissipation and maintain uniform cooling at altitude. This doubles power density while reducing hardware weight by 50–75%. KPI: 2× power density, up to 50% component count reduction, & ≥25% lower temperature. |
| Streamlined Propulsion System Integration TRL Start: 3 TRL Finish: 6 | Current aerospace powertrains are fragmented: motors, inverters, & cooling subsystems are packaged separately, linked by heavy wiring harnesses. This causes redundant hardware, increased electromagnetic interference, and penalties in weight and efficiency. The challenge is to integrate these components into a lightweight modular system without sacrificing performance. | Evolito, Helix, and others demonstrate motor–drive integration, but cooling loops and wiring harnesses are typically external. This adds 15–20% redundant weight, limits system-level power density, and complicates thermal management. Integration is often component-level rather than system-level, preventing significant mass savings. | SkyDrive integrates motor, inverter, and cooling into a single modular housing, with additive-manufactured channels providing direct liquid cooling. This reduces cabling mass by 20–40% and overall propulsion mass by ≥10%, while achieving scalability from 100 kW to MW-class systems. Crucially, integration is validated on ARC Aerosystems’ jump take-off (eJTO) platform, which demonstrates the propulsion unit under real aircraft installation conditions. This proves both compact integration and operational performance in a relevant aerospace environment. KPI: ≥10% propulsion system mass reduction, 20–40% cabling weight reduction, and system-level validation on ARC’s eJTO platform. |
| AI-Powered Fault Tolerant Control and Validation TRL Start: 4 TRL Finish: 6 | Reliability remains a barrier: conventional control systems rely on deterministic redundancy, meaning failure of one controller or module can still disable propulsion. Testing is fragmented, with limited ability to validate under real aerospace conditions (altitude, vibration, pressure). | Aerospace demonstrators typically use fixed backup channels and hardware redundancy. Maintenance is reactive, and validation rigs are industrial-grade, unable to replicate flight stresses. ARC-level aerospace test benches are rare, leaving a gap in predictive validation. | SkyDrive combines FPGA-based closed-loop control with AI-driven digital twins to predict and reconfigure under fault conditions. Validation is conducted on ARC Aerosystems’ jump take-off (eJTO) aerospace-grade platform, which replicates altitude, pressure, and dynamic load conditions. This enables predictive reliability testing, real-world verification, and accelerates TRL progression. KPI: 100% improvement in fault tolerance, TRL 3 → 6 validation, and predictive validation using ARC Aerosystems’ eJTO platform. |

9.2 List of KPIs

| Project Internal Key Performance indicators (Technical) | Target |
| --- | --- |
| KPI 1. Reduction in propulsion system mass by optimising motor-drive integration and eliminating redundant wiring. | ≥10% mass reduction |
| KPI 2. Improvement in electric motor power density through lightweight materials, high-speed design, and advanced cooling. | 13 kW/kg |
| KPI 3. Enhancement of system fault tolerance via multiphase motor and inverter architecture enabling continued operation under phase loss. | 100% improvement vs. SoA |
| KPI 4. Reduction of electromagnetic interference and voltage stress through multilevel inverter topology and optimised switching control. | 50–70% EMI reduction |
| KPI 5. Enhancement of thermal management using additive-manufactured microchannel cooling and hollow conductors to reduce hot spots. | ≥25% lower temperature |
| KPI 6. Improvement in inverter and power electronics compactness by reducing filter size, component count, and packaging volume. | Up to 50% reduction |
| KPI 7. Increase in overall system power density by integrating electric motor and power electronics within a single compact housing. | 16 kW/kg |
| KPI 8. Validation of propulsion system under aerospace-grade test bench conditions with digital twin predictive capability. | TRL 3 → 6 |
| KPI 9. Demonstrated improvement in aircraft range and payload capacity enabled by more efficient and lighter powertrain architecture. | +10–15% range/payload |

9.3 Requirements Based Engineering and Technology Selection Approach

Figure 1. SkyDrive architecture

The propulsion architecture, as shown in Figure 1, is defined directly from the operational requirements of ARC Aerosystems’ LINX P3 platform, which demands lightweight construction, compact nacelle packaging, reliable performance under altitude conditions, and fault tolerance suitable for distributed propulsion. These requirements form the primary input into the engineering process, ensuring traceability from top-level aircraft needs to propulsion subsystem specifications. Several alternative pathways are assessed, including conventional three-phase motor–drive units with external cooling, axial-flux machines with hybrid air–water systems, and tightly integrated multiphase machines with embedded inverters and advanced thermal management. Comparative analysis demonstrates that only an integrated motor–drive–cooling architecture can meet the weight, scalability, and operational reliability constraints set by the P3.

A baseline specification is defined to guide design iterations: a continuous 100 kW electric motor operating at 1 kV and 18,000 rpm, combined with a multilevel inverter topology and embedded cooling. Key design objectives include achieving high gravimetric density, sustaining torque output under partial faults, suppressing EMI and voltage overshoot, and ensuring uniform heat extraction at altitude. These baseline requirements set the parameter ranges for electromagnetic design, winding technology, insulation strategies, inverter architecture, and thermal solutions.

Candidate technologies are evaluated through an iterative process combining high-fidelity electromagnetic, thermal, and structural simulations with sensitivity studies of parasitic inductance, insulation stress, and thermal gradients. This process identifies which combinations of motor topology, inverter design, and cooling approach remain compatible with the baseline, while systematically eliminating architectures that impose excessive weight penalties or fail to maintain altitude resilience. Rapid prototyping and bench-level assessments are employed to refine material selection, winding fill, sleeve reinforcement, and microchannel cooling geometries.

The methodology applies model-based systems engineering principles supported by structured performance matrices. These matrices capture design objectives and allow relative ranking of motor and drive technologies against gravimetric density, reliability, EMI suppression, and thermal uniformity. Maturity is tracked through progressive TRL stages, beginning at TRL 3 for conceptual design and advancing through subsystem demonstration (TRL 4), integrated bench validation (TRL 5), and aerospace-grade testing at TRL 6.

Requirement variability is addressed by adopting the P3’s most demanding operating scenarios, including jump take-off and steep climb profiles, as the design driver. By targeting these critical cases, the propulsion package is inherently robust to less demanding applications. At the same time, modularity in motor topology, inverter structure, and thermal architecture ensures scalability to both smaller eVTOL platforms and larger MW-class regional aircraft. Validation on ARC Aerosystems’ jump take-off platform provides final assurance that the selected technologies perform as intended in representative aerospace environments.

9.4 Technical Description of Challenges and Solutions for the Electrical Machine (Led by iNetic)

iNetic will study advanced multiphase permanent magnet motor architectures tailored to the ARC Aerosystems LINX P3 platform. Both six-phase and dual three-phase configurations will be evaluated through electromagnetic and system-level studies. Comparative analysis will quantify behaviour under healthy and faulty operation. Dual three-phase windings will be assessed for their ability to achieve magnetic motive force harmonic cancellation and reduced saturation, which improves degraded mode performance. Six-phase windings will be examined for their ability to distribute current evenly across multiple phases, increasing redundancy. Both configurations will be modelled with concentrated winding layouts, reducing harmonic distortion and copper losses while localising faults so that failure in one set does not affect the others.

| (a) multi-phase motor | (b) 6-phase winding | (c) dual-3phase winding | (d) healthy and faulty performance |
| --- | --- | --- | --- |

Figure 4. Preliminary study on the multi-phase electric machine

The stator will include engineered cavities to manage insulation stresses and limit fault propagation. These cavities will also be optimised for improved thermal performance. Hollow copper conductors will be introduced in winding designs to enable direct liquid cooling of the slots. Spray cooling will be applied to permanent magnets and stator regions with high local heat flux. Additive-manufactured thermal jackets with integrated microchannels will be developed to achieve uniform fluid distribution and suppress hotspots across the housing and end-windings.

Electromagnetic studies will incorporate advanced materials. Silicon cobalt laminations (Vacoflux) will be used to reduce iron losses and increase flux density capability. Metglas amorphous laminations will be assessed for their low hysteresis loss and high saturation flux density, improving efficiency in high-frequency regimes. Rotor mechanical studies will apply carbon fibre reinforced sleeves, including T1000G technology developed by iNetic, to secure magnets against centrifugal forces and ensure rotor stability at high speeds.

To adapt motor speed to the propulsor requirements, a compact planetary gearbox will be designed in conjunction with the motor. The gearbox design will be optimised for efficiency and packaging compactness, ensuring minimal impact on overall system mass while matching the speed–torque envelope required for the P3.

All aspects of the design will be evaluated through iterative analytical methods, high-fidelity finite element modelling, and multiphysics CFD analysis. Electromagnetic, structural, and thermal-fluid domains will be studied in a coupled manner to capture interactions such as harmonic-induced heating, rotor stress, and coolant flow. Results from these simulations will inform iterative refinements, with targeted experimental validation at subcomponent and prototype level. This process will ensure that the selected motor configuration achieves the required performance while maintaining resilience under faulty conditions and scalability to aerospace propulsion requirements.

9.5 Technical Description of Challenges and Solutions for Power Electronics (Led by UCL)

The design of a control-integrated chip will be the foundation of UCL’s power electronics innovation. This chip will directly sequence cascaded voltage modules in a precisely coordinated manner, ensuring that the output voltage is built up by adding or subtracting small voltage steps rather than switching a single high-voltage device at high frequency. By controlling the timing of each cell through the integrated circuit, the output waveform will be smooth and stable, parasitic inductances will be minimised, and electromagnetic interference will be drastically reduced. This method avoids the abrupt dv/dt associated with conventional PWM modulation, removing the dominant source of insulation stress and allowing reliable operation at aerospace voltages.

In this architecture, the power converter will no longer rely on fast, high-voltage PWM switching. Instead, the total output is synthesised from many low-voltage, low-stress increments that can be switched at modest frequencies. Each cascaded H-bridge cell contributes a small voltage increment, and the integrated chip manages their sequencing so that the combined output is effectively sinusoidal. Because each transition is small, the resulting waveform has low harmonic distortion and requires no large external inductors or capacitors for filtering. This eliminates bulky magnetic components and heavy shielding, enabling a power converter that is significantly smaller, lighter, and more efficient than state-of-the-art aerospace systems. The integrated chip ensures tight synchronisation of the cells, prevents imbalance between modules, and provides a pathway for modular scaling from 100 kW-class systems to MW-class propulsion.

The removal of high-frequency PWM switching at high voltage also has a direct impact on reliability. With dv/dt stresses suppressed, partial discharge and insulation degradation in the machine windings are avoided, and common-mode currents are minimised. The architecture therefore extends insulation life, reduces electromagnetic interference across the aircraft electrical system, and improves overall system robustness. Furthermore, integrating the control directly into the chip eliminates parasitic loops from discrete gate-driver connections and ensures that high-speed operation of wide-bandgap devices can be achieved without instability.

Thermal management will complement this architecture through the use of microchannel cooling embedded directly in the substrate. Power semiconductor modules will be packaged with high-aspect-ratio microchannels fabricated using additive manufacturing, creating a dense network of flow paths in direct contact with the heat-generating junctions. This approach allows highly uniform fluid distribution, reducing thermal gradients and eliminating local hotspots that often limit converter lifetime. The use of dielectric coolant will ensure electrical safety while enabling high heat flux dissipation at altitude, where reduced air density limits the effectiveness of conventional cold-plate cooling. By embedding the cooling structure into the power module packaging itself, thermal resistance is minimised, supporting continuous high-power operation without derating.

All design tasks will be carried out iteratively, beginning with analytical studies of cascaded module sequencing, followed by high-fidelity circuit simulations and multiphysics analysis of electrical and thermal performance. Hardware prototypes will be developed to validate the behaviour of the integrated chip, confirm waveform quality, and verify thermal performance under representative load conditions. This workflow ensures that the converter developed by UCL will deliver an ultra-compact, highly efficient, and fault-tolerant solution specifically tailored for ARC Aerosystems’ LINX P3 propulsion platform.

9.6 System Testing and Validation on the ARC eJTO Platform (Led by ARC Aerosystems)

ARC Aerosystems will lead the testing and validation of the integrated motor–drive system using its jump take-off (eJTO) platform. This platform provides a unique aerospace-grade environment where propulsion technologies can be subjected to realistic operational conditions, including altitude variation, pressure cycles, vibration, and dynamic loading. The eJTO testbed replicates the mission profiles of the LINX P3 aircraft, allowing the propulsion unit to be evaluated under demanding phases such as take-off, climb, and rapid power transients.

Validation at the eJTO facility will focus on three critical aspects. First, the multiphase motor and integrated inverter will be tested for fault-tolerant operation, confirming continued torque delivery under simulated phase and inverter faults. Second, the propulsion unit will be monitored for thermal performance, with embedded sensors verifying the effectiveness of hollow conductors, spray cooling, and microchannel substrates in managing hotspots at altitude. Third, the system will undergo EMI and voltage stress assessment, where the performance of the multilevel converter and integrated control chip will be benchmarked against aerospace compliance requirements.

Testing will be carried out iteratively, beginning with subsystem verification on dynamometers and progressing to full nacelle-level integration on the eJTO demonstrator. Digital twin models will run in parallel with physical testing to correlate predictions from electromagnetic, thermal, and structural simulations with measured results. This approach enables rapid refinement of design models, shortens the development cycle, and builds confidence in the transition from TRL 4 to TRL 6.

The eJTO validation programme not only de-risks the integration of the propulsion system into ARC’s LINX P3 platform but also establishes a transferable framework for certifying future electric propulsion technologies. By combining laboratory-grade monitoring with real-world operating conditions, the eJTO test campaign will provide definitive evidence of performance, safety, and reliability, ensuring the system is ready for deployment in next-generation aerospace applications.